WEEK 2.1 NETWORK ANALYSIS REPORT

Introduction

This report represents a network analysis of the national electricity grid using graph theory. Substations were modelled as nodes and transmission lines as edges using the python library NetworkX. The analysis examined substation connectivity, critical bridge points, regional clustering, and network resilience.

Methodology

The clean substation and line datasets were used to build an undirected graph, with substations as nodes and lines as edges. Network metrics were then calculated, including degree and betweenness centrality, community detection, shortest-path analysis, and N-1 contingency test.

Critical Substation Analysis

Degree centrality measures how many direct connections each substation has. Substations 3,7, and 16 were the most connected, each connected to other 5 substations.

Centrality Analysis

Betweenness centrality identifies substations that frequently sit on the shortest path between other substations, acting as bridges. Substation 16 recorded the highest score (0.525), followed by 12 (0.516). Notably, substation 12 had only moderated degree centrality, showing that a substation does not need many direct connections to be structurally important.

Community Detection

Using the greedy modularity algorithm, the network was found to contain 10 distinct communities, closely matching Ghana’s real administrative regions. This indicates that electricity network is structurally organised along regional lines, even though labels were not in clustering process.

N-1 Contingency Analysis

To test network resilience, high- and low-centrality substations were individually removed from the graph. Removing Substation 16 (Highest betweenness) increased the number of components from 3 to 5, while removing Substation 1 (Lower betweenness) increased it from 3 to 4. This confirms that betweenness centrality is a meaningful predictor of network fragility: Substations with higher betweenness scores cause greater fragmentation when removed.

Conclusion

The analysis identified Substations 3, 7 and 16 as key local hubs and substation 16 specifically as the most structurally critical bridge in the network. Community detection showed that the grid is naturally organised by region, and path analysis revealed that two substations are entirely disconnected from the main network. The N-1 contingency test confirmed that losing high betweenness substations poses a greater risk to overall connectivity than losing lower centrality-ones. These findings are based on graph structure alone and should be interpreted as proxies for reliability rather than confirmed operational risk, since they do not account for real electrical load, protection systems, or power-flow behaviour.